Sunseeker Driver Controls

Switches

Electrically Isolated

- Master Relay On/Off -  DPST for Power Relays

- Six MPPT On/Off, SPST

- Four FAN On/Off - Driver Compartment, 
Note: Battery Fan Always on, turned off by disconnecting

- Momentary Switch for Battery Protection System (BPS) Reset

- Momentary Switch auxiliary

- Horn On/Off

Driver Controller Related

- Key On/CAN Devices Operational – On/Off

- 3-position toggle turn signal – Right-Neutral-Left

- Brake On/Off – remote contact switch

- Hazard Lights On/Off

Rotary Shaft Encoder

- Cruise Control Velocity

Analog Inputs

- Accelerator/Regenerative Braking

Displays

Numerical Displays

- Two 3.5 Digit LCD displays, 4 modes for each display, CAN Bus Ribbon connection

LED Displays

- Right Turn Indicator,

- Left Turn Indicator

- Braking Indicator

2008 Connections

Sunseeker Driver Controls

Enc1 POT In (3): Analog 1 Motor current set point, connect to accelerator (gas) pedal position

Enc1 A In (1)

Enc1 B In (20)

Enc1 SW In (2)

Enc1 LED Out (21): Analog 1 V+ Output (330 ohm series to uC)

Ground (22, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39)

Enc2 POT In (6): Analog 2 Velocity set point

Enc2 A In (4)

Enc2 B In (23)

Enc2 SW In (5)

Enc2 LED Out (24): Analog 2 V+ Output (330 ohm series to uC)

RH Ind. In (7) - Switch 1 Right-hand indicator (blinker)

LH Ind. In (8) - Switch 2 Left-hand indicator (blinker)

Hazard In (9) - Switch 3 Hazards (4-way blinker)

Horn In (10) - Switch 4 Horn

Ign Acc In (11) - Switch 5 Ignition 'accessories' position

Ign On In (12) - Switch 6 Ignition 'run' position (vehicle enable)

Direction In (13) - Switch 7 Forward / Reverse

Brakes 1 In (14) - Switch 8 Brake switch / pedal 1

Brakes 2 In (15) - Switch 9 Brake switch / pedal 2 (redundant spare)

Lights Park In (16) - Switch 10 Lights – side / running lights

Lights Low In (17) - Switch 11 Lights – headlights low beam

Lights High In (18) - Switch 12 Lights – headlights high beam

+12 V IO (19) – Schottky Diode Drop from 13.8 V