# Modem Initialization and Operation

# General Modem Comments

The input data buffer is 0x800 bytes or 2048 bytes.

The default RS-232/485 baud rate is 9600.

# Message Packets

Header:  8 bytes

char pre_msg[8];// = "ABCDEF\r\n";

Message element: 30 bytes

char message[MSG_SIZE];// = "XXXXXX,0x00000000,0x00000000\r\n";

Time element: 17 bytes

char time_msg[17];// = "TL_TIM,HH:MM:SS\r\n";

Footer: 8 bytes and null character to end transmission

char post_msg[9];// = "UVWXYZ\r\n\0";

Total Bytes

High Rate Packet: 8 + 15 x 30 + 17 + 8 = 483 bytes (0x1E3)

Low Rate Packet: 8 + 16 x 30 + 17 + 8 = 513 bytes (0x201)

Status Rate Packet: 8 + 12 x 30 + 17 + 8 = 393 bytes (0x189)

# Entering command mode

To enter the command mode, wait for 1 sec., send “+++”, wait for 1 sec., and the device should be in the command mode.

## Sunseeker MSP430 Telemetry Programming

The modem requires network communication initialization to establish the communication structure and addressing. In addition, in order for an acknowledge packet stream to occur, a unique destination address must be established.

ATAM <Enter>	;	AM (Auto-set MY) Command

ATMY 786<Enter>	;	MY (Source Address) Command using hex address 786

ATDT 786<Enter>	;	DT (Destination Address) Command using hex address 786

Communication is performed using packets of information. The packet size for transmission and retransmission must be known. After packet transmission a minimal delay is required to receive an acknowledgment before retransmission occurs. The modem will not send a packet until either a predefined number of serial characters are written or no serial characters are received for a timeout period.

ATRR 3<Enter>	;	RR (Retries) Command allowing up to 3 retransmissions

ATRN 4<Enter>	;	RN (Delay Slots) Command allowing from 0 to 3 x 5 msec delay for ACK

ATPK 447<Enter>	;	PK (Maximum RF Packet Size) Command in hex, 513 characters is 201 hex

ATRB 189<Enter>	;	RB (Packetization Threshold) Command before TX begins less than or equal to PK

ATRO 128<Enter>	;	RO (Packetization Timeout) Command in 296 UART char times (~0.37 sec) before TX if RB not reached (too few chars in packet for some reason)

To minimize modem power, the serial port sleep mode will be used. When there are no serial transfers for a defined period of time, the modem will enter a sleep mode. When waking up from a sleep mode, if an extended period has gone by, a wake-up initialization sequence of a defined length will be sent before the data packet. As a note, once sleep mode is entered, the modem will not receive RF communications.

ATSM 2<Enter>	;	SM (Sleep Mode) Command for the serial port sleep mode

ATAT 2<Enter>	;	AT (Guard Time After) Command using a minimum 0.2 sec of inactivity so that ATST can be set to it’s minimum value

ATST 5<Enter>	;	ST (Time before Sleep) Command using a minimum 0.5 sec of inactivity

ATLH 2<Enter>	;	LH (Wake-up Initializer Timer) Command allowing a 0.2 sec initialization

ATHT 258<Enter>	;	HT (Time before Wake-up Initializer) Command causing a wake-up init to occur only after the sleep mode lasts for more than 60 sec. (600 x 0.1).

The modem can transmit using a range of power. The default is 1 watt, which is quite high for most applications and requires significant transmitter power. To reduce this a 100 mW transmission mode will be used.

ATPL 2<Enter>	;	PL (TX Power Level) Command for 100mW transmission power

To return the device to a normal mode of operation, you can either wait for 20 sec. (default ATCT value) or execute the ATCN command.

ATCN<Enter>	;	CN (Exit AT Command Mode) Command

## Serial Link Baud Rate Change Notes

The serial port baud rate may be increased. The default rate is 9600 bits per second. For the data transfers being performed, there is 1 start bit, 8 data bits and 2 stop bits or 11 bits per character. This rate is equivalent to 872.7 characters per second. It should be possible to send either 19200 or 38400 bps to achieve, respectively, 1745 or 3490.9 char per sec. The commands to increase the rate are

ATBD 4<Enter>	;	BD (Interface Data Rate) Command to standard rate 4 -> 19200 bps

ATBD 5<Enter>	;	BD (Interface Data Rate) Command to standard rate 5 -> 38400 bps

ATBD 6<Enter>	;	BD (Interface Data Rate) Command to standard rate 6 -> 57600 bps

ATBD 7<Enter>	;	BD (Interface Data Rate) Command to standard rate 7 -> 115200 bps

If this is to be performed, use the 16 MHz clock output from the CAN controller and connect it to the XT2IN input pin. Using this as a direct rate, 9600 requires N=1667, 19.2k requires N=833, 38.4k requires N=417, 57.6 requires N=278, and 115.2k requires N=139.

Note: modem handshaking will be required to perform this task. So far, the software attempts to change the baud rate have failed ( send “+++” receive the OK<CR> and then adjust the rate). A coupled problem will be the power reset of the modem for the next time it is used.

# Programming Synopsis

## Sunseeker telemetry PCB initialization

static char RFModemH[76] = "ATAM,MY 786,DT 786\rATRR 3,RN 4\r

ATPK 201,RB 1E3,RO 128\rATPL 2\rATBD 3\rATCN\r\0\0";

static char RFModemL[76] = "ATAM,MY 786,DT 786\rATRR 3,RN 4\r

ATPK 201,RB 201,RO 128\rATPL 2\rATBD 3\rATCN\r\0\0";

static char RFModemS[76] = "ATAM,MY 786,DT 786\rATRR 3,RN 4\r

ATPK 201,RB 189,RO 128\rATPL 2\rATBD 3\rATCN\r\0\0";

## Chase vehicle receiver initialization

ATAM, MY 786, DT 786<Enter>

ATCN<Enter>